Ehrlich: Patient Care in Radiography, 8th Edition

Chapter 01: Introduction to Radiography

Test Bank

MULTIPLE CHOICE

1.	X-rays were discovered in 1895 by:

| a. | Coolidge. |
| --- | --- |
| b. | Crookes. |
| c. | Roentgen. |
| d. | Edison. |

ANS:	C 	REF:	p. 3	OBJ:	1

2.	Of the following types of electromagnetic energy, which has the shortest wavelength?

| a. | Radio waves |
| --- | --- |
| b. | X-rays |
| c. | Visible light |
| d. | Ultraviolet light |

ANS:	B 	REF:	p. 8	OBJ:	8

3.	Which of the following is NOT an accurate statement regarding the characteristics of x-rays?

| a. | They can penetrate matter that is impenetrable to light. |
| --- | --- |
| b. | They cause certain crystals to fluoresce. |
| c. | They can be refracted by a lens. |
| d. | They cannot be detected by the human senses. |

ANS:	C 	REF:	p. 9	OBJ:	8

4.	An electron cloud surrounding a hot cathode is referred to as a(n):

| a. | focusing cup. |
| --- | --- |
| b. | ampere. |
| c. | space charge. |
| d. | filament. |

ANS:	C 	REF:	p. 7	OBJ:	4

5.	The purpose of rotating the x-ray tube target is to:

| a. | create a space charge. |
| --- | --- |
| b. | remove long-wavelength photons from the x-ray beam. |
| c. | focus the electron stream on a small target area. |
| d. | increase the heat capacity of the anode. |

ANS:	D 	REF:	p. 12	OBJ:	4

6.	An imaginary photon that is emitted from the center of the focal spot, perpendicular to the long axis of the x-ray tube, is called the:

| a. | electron stream. |
| --- | --- |
| b. | x-ray beam. |
| c. | central ray. |
| d. | radiation field. |

ANS:	C 	REF:	p. 13	OBJ:	6

7.	A device used to control the size of the radiation field is a:

| a. | collimator. |
| --- | --- |
| b. | detent. |
| c. | tube housing. |
| d. | filter. |

ANS:	A 	REF:	p. 13	OBJ:	11

8.	The function of an x-ray grid is to:

| a. | decrease patient radiation dose. |
| --- | --- |
| b. | increase radiographic resolution. |
| c. | increase radiographic contrast by reducing scatter radiation fog. |
| d. | increase radiographic density. |

ANS:	C 	REF:	p. 15	OBJ:	11

9.	The majority of radiography education programs today are based in:

| a. | proprietary schools. |
| --- | --- |
| b. | hospitals. |
| c. | clinics. |
| d. | colleges. |

ANS:	D 	REF:	p. 5	OBJ:	3

10.	The container for the vacuum of the x-ray tube is made of:

| a. | glass. |
| --- | --- |
| b. | aluminum. |
| c. | tungsten. |
| d. | carbon. |

ANS:	A 	REF:	p. 7	OBJ:	4

11.	X-rays are a type of:

| a. | electricity. |
| --- | --- |
| b. | microwave. |
| c. | kinetic energy. |
| d. | electromagnetic energy. |

ANS:	D 	REF:	p. 7	OBJ:	8

12.	The frequency of an electromagnetic sine wave is defined as:

| a. | the distance from trough to crest. |
| --- | --- |
| b. | the distance from one crest to the next. |
| c. | the number of times per second that a crest passes a given point. |
| d. | the velocity at which the wave travels through space. |

ANS:	C 	REF:	p. 8	OBJ:	9

13.	Which of the following formulas represents the relationship between the wavelength, frequency, and velocity of an electromagnetic wave?

| a. | f = λ × v |
| --- | --- |
| b. | V = λ × f |
| c. | Λ = f ÷ v |
| d. | f = λ ÷ v |

ANS:	B 	REF:	p. 8	OBJ:	9

14.	Which of the following substances is most readily penetrated by x-rays?

| a. | Water |
| --- | --- |
| b. | Air |
| c. | Bone |
| d. | Fat |

ANS:	B 	REF:	p. 9	OBJ:	8

15.	Grids or buckys are generally used for body parts that measure greater than:

| a. | 2 to 4 cm. |
| --- | --- |
| b. | 10 to 12 cm. |
| c. | 18 to 20 cm. |
| d. | 30 cm. |

ANS:	B 	REF:	p. 17	OBJ:	11

16.	The access point for the radiographer to determine the exposure factors and to initiate the exposure is called the:

| a. | transformer. |
| --- | --- |
| b. | image receptor unit. |
| c. | control console. |
| d. | stationary grid. |

ANS:	C 	REF:	p. 17	OBJ:	11

17.	An x-ray machine designed for direct viewing of the x-ray image is called a(n):

| a. | image receptor. |
| --- | --- |
| b. | transformer. |
| c. | control console. |
| d. | fluoroscope. |

ANS:	D 	REF:	p. 19	OBJ:	11

18.	A device located between the x-ray tube and the control panel that increases the voltage delivered from the power company is called a:

| a. | collimator. |
| --- | --- |
| b. | transformer. |
| c. | control console. |
| d. | fluoroscope. |

ANS:	B 	REF:	p. 17	OBJ:	11

19.	The anode or positive end of the x-ray tube is the end that contains the:

| a. | target. |
| --- | --- |
| b. | filament. |
| c. | focusing cup. |
| d. | space charge. |

ANS:	A 	REF:	p. 7	OBJ:	5

20.	The inventor of the fluoroscope, who also investigated hundreds of fluorescent materials, was:

| a. | Coolidge. |
| --- | --- |
| b. | Pupin. |
| c. | Roentgen. |
| d. | Edison. |

ANS:	D 	REF:	p. 4	OBJ:	2

21.	The inventor of the hot cathode x-ray tube, the prototype of the x-ray tubes of today, was:

| a. | Coolidge. |
| --- | --- |
| b. | Pupin. |
| c. | Roentgen. |
| d. | Edison. |

ANS:	A 	REF:	p. 4	OBJ:	2