Hedrick: Technology for Diagnostic Sonography

Chapter 1: Properties of Sound Waves

Test Bank

MULTIPLE CHOICE

1.	What is the wavelength in soft tissue for a 10-MHz ultrasonic wave?

| a. | 0.15 cm | c. | 0.30 cm |
| --- | --- | --- | --- |
| b. | 0.15 mm | d. | 0.30 mm |

ANS:	B

Determined by the relationship between acoustic velocity, frequency, and wavelength

PTS:	1	REF:	6

OBJ:	Know the relationship between acoustic velocity, frequency, and wavelength.

TOP:	Properties of sound waves

2.	What is the wavelength in fat for a 5-MHz ultrasonic wave?

| a. | 0.14 cm | c. | 0.29 cm |
| --- | --- | --- | --- |
| b. | 0.14 mm | d. | 0.29 mm |

ANS:	D

Determined by the relationship between acoustic velocity, frequency, and wavelength

PTS:	1	REF:	6

OBJ:	Know the relationship between acoustic velocity, frequency, and wavelength.

TOP:	Properties of sound waves

3.	What is the period for a 5-MHz ultrasonic wave?

| a. | 0.2 ns | c. | 0.2 ms |
| --- | --- | --- | --- |
| b. | 0.2 s | d. | 0.2 s |

ANS:	B

Determined by the relationship between frequency and period

PTS:	1	REF:	4

OBJ:	Know the relationship between frequency and period.	TOP:	Properties of sound waves

4.	What is the period for a 10-MHz ultrasonic wave?

| a. | 0.10 s | c. | 0.20 s |
| --- | --- | --- | --- |
| b. | 0.15 s | d. | 0.30 s |

ANS:	A

Determined by the relationship between frequency and period

PTS:	1	REF:	4

OBJ:	Know the relationship between frequency and period.	TOP:	Properties of sound waves

5.	What term describes the region of increased molecular density induced by sound propagation in a medium?

| a. | Compression | c. | Fresnel |
| --- | --- | --- | --- |
| b. | Focal | d. | Rarefaction |

ANS:	A

Pressure variations produce alternating regions of high and low particle density.

PTS:	1	REF:	1-2	OBJ:	Describe the propagation of sound.

TOP:	Properties of sound waves

6.	What is the compression zone?

| a. | Region of high particle density | c. | Region of high acoustic impedance |
| --- | --- | --- | --- |
| b. | Region of low particle density | d. | Region of low acoustic impedance |

ANS:	A

Pressure variations produce alternating regions of high and low particle density.

PTS:	1	REF:	1-2	OBJ:	Describe the propagation of sound.

TOP:	Properties of sound waves

7.	What term describes the region of decreased molecular density induced by sound propagation in a medium?

| a. | Compression | c. | Fresnel |
| --- | --- | --- | --- |
| b. | Focal | d. | Rarefaction |

ANS:	D

Pressure variations produce alternating regions of high and low particle density.

PTS:	1	REF:	1-2	OBJ:	Describe the propagation of sound.

TOP:	Properties of sound waves

8.	What is the rarefaction zone?

| a. | Region of high particle density | c. | Region of high acoustic impedance |
| --- | --- | --- | --- |
| b. | Region of low particle density | d. | Region of low acoustic impedance |

ANS:	B

Pressure variations produce alternating regions of high and low particle density.

PTS:	1	REF:	1-2	OBJ:	Describe the propagation of sound.

TOP:	Properties of sound waves

9.	What term describes the number of pressure oscillations per unit of time?

| a. | Acoustic velocity | c. | Period |
| --- | --- | --- | --- |
| b. | Frequency | d. | Wavelength |

ANS:	B

Definition of frequency

PTS:	1	REF:	4	OBJ:	Know the definition of frequency.

TOP:	Properties of sound waves

10.	What term describes the distance between compression zones?

| a. | Acoustic velocity | c. | Period |
| --- | --- | --- | --- |
| b. | Frequency | d. | Wavelength |

ANS:	D

Definition of wavelength.

PTS:	1	REF:	3	OBJ:	Know the definition of wavelength.

TOP:	Properties of sound waves

11.	What term describes the distance of one cycle for a wave?

| a. | Acoustic velocity | c. | Period |
| --- | --- | --- | --- |
| b. | Frequency | d. | Wavelength |

ANS:	D

Definition of wavelength.

PTS:	1	REF:	3	OBJ:	Know the definition of wavelength.

TOP:	Properties of sound waves

12.	What is the frequency for the ultrasound wave, which has a wavelength of 0.20 mm in soft tissue?

| a. | 10 MHz | c. | 6 MHz |
| --- | --- | --- | --- |
| b. | 7.5 MHz | d. | 3.5 MHz |

ANS:	B

Determined by the relationship between acoustic velocity, frequency, and wavelength

PTS:	1	REF:	6

OBJ:	Know the relationship between acoustic velocity, frequency, and wavelength.

TOP:	Properties of sound waves

13.	Which material has the lowest acoustic velocity?

| a. | Bone | c. | Muscle |
| --- | --- | --- | --- |
| b. | Fat | d. | Air |

ANS:	D

Determined by the bulk modulus and density of the medium

PTS:	1	REF:	6	OBJ:	Know the relative velocity for various materials.

TOP:	Properties of sound waves

14.	In which material does a 10-MHz sound wave have the longest wavelength?

| a. | Bone | c. | Muscle |
| --- | --- | --- | --- |
| b. | Fat | d. | Air |

ANS:	A

Material with the greatest acoustic velocity has the longest wavelength.

PTS:	1	REF:	6

OBJ:	Know the relationship between acoustic velocity, frequency, and wavelength.

TOP:	Properties of sound waves

15.	What is the period of an ultrasound wave whose frequency is 5 MHz?

| a. | 0.002 s | c. | 3.1 x 10-4 s |
| --- | --- | --- | --- |
| b. | 5 x 106 s | d. | 2.0 x 10-7 s |

ANS:	D

Determined by the relationship between frequency and period

PTS:	1	REF:	4

OBJ:	Know the relationship between frequency and period.	TOP:	Properties of sound waves

16.	Which factor affects the acoustic velocity in a medium?

| a. | Density of the medium |
| --- | --- |
| b. | Cross-sectional area of ultrasound beam |
| c. | Wavelength |
| d. | Frequency |

ANS:	A

Acoustic velocity is determined by the bulk modulus and density of the medium.

PTS:	1	REF:	5	OBJ:	Identify the factors that affect acoustic velocity.

TOP:	Properties of sound waves

17.	What characterizes the elasticity of a medium?

| a. | Fractional change is shape from an applied force |
| --- | --- |
| b. | Return to original shape after removal of applied force |
| c. | High particle velocity during propagation |
| d. | Low particle velocity during propagation |

ANS:	B

Elasticity describes the ability of a medium to return to its original shape when a force is removed.

PTS:	1	REF:	5	OBJ:	Describe the propagation of sound.

TOP:	Properties of sound waves

18.	What denotes a longitudinal wave?

| a. | Particle motion is along the direction of propagation |
| --- | --- |
| b. | Particle motion is perpendicular to the direction of propagation |
| c. | Wavelength over 1 mm |
| d. | Wavelength over 1 cm |

ANS:	A

Attribute of longitudinal wave

PTS:	1	REF:	3	OBJ:	Describe the propagation of sound.

TOP:	Properties of sound waves

19.	What describes a transverse wave?

| a. | Particle motion is along the direction of propagation |
| --- | --- |
| b. | Particle motion is perpendicular to the direction of propagation. |
| c. | Long wavelength over 1 mm |
| d. | Long wavelength over 1 cm |

ANS:	B

Attribute of transverse wave

PTS:	1	REF:	3	OBJ:	Describe the propagation of sound.

TOP:	Properties of sound waves

20.	Which type of wave exhibits particle motion along the direction of propagation?

| a. | Longitudinal | c. | Transverse |
| --- | --- | --- | --- |
| b. | Sheer | d. | Light |

ANS:	A

Attribute of longitudinal wave

PTS:	1	REF:	3	OBJ:	Describe the propagation of sound.

TOP:	Properties of sound waves

21.	What term describes the rate of flow of energy over a small area?

| a. | Compression | c. | Pressure |
| --- | --- | --- | --- |
| b. | Intensity | d. | Frequency |

ANS:	B

Intensity describes the transmittal of energy by the sound wave.

PTS:	1	REF:	5	OBJ:	Describe the propagation of sound.

TOP:	Properties of sound waves

22.	What are the units for intensity?

| a. | J/s | c. | Nt/cm2 |
| --- | --- | --- | --- |
| b. | m/s | d. | mW/cm2 |

ANS:	D

Intensity has units of energy per time (mW) per area (cm2).

PTS:	1	REF:	5	OBJ:	Know the definition of intensity.

TOP:	Properties of sound waves

23.	What is the period for the ultrasound wave, which has a wavelength of 0.20 mm in soft tissue?

| a. | 0.10 s | c. | 0.20 s |
| --- | --- | --- | --- |
| b. | 0.13 s | d. | 0.24 s |

ANS:	B

Calculate frequency using the acoustic velocity in tissue and take the reciprocal to determine the period.

PTS:	1	REF:	4

OBJ:	Know the relationship between frequency and period.	TOP:	Properties of sound waves

24.	What is the frequency range for human hearing?

| a. | 20 to 20,000 Hz | c. | 20 to 1500 Hz |
| --- | --- | --- | --- |
| b. | 20 to 20,000 kHz | d. | 20 to 1500 kHz |

ANS:	A

The frequency range for human hearing is 20 to 20,000 Hz.

PTS:	1	REF:	3	OBJ:	Know the frequency range for human hearing.

TOP:	Properties of sound waves

25.	What is the lower frequency limit for ultrasound?

| a. | 20 Hz | c. | 2000 Hz |
| --- | --- | --- | --- |
| b. | 200 Hz | d. | 20,000 Hz |

ANS:	D

Ultrasound is sound with a frequency above 20,000 Hz.

PTS:	1	REF:	3	OBJ:	Know the frequency range for ultrasound.

TOP:	Properties of sound waves

26.	Which acoustic variable is equal to acoustic velocity divided by the frequency?

| a. | Intensity | c. | Period |
| --- | --- | --- | --- |
| b. | Pressure | d. | Wavelength |

ANS:	D

The acoustic velocity and frequency establish the wavelength via c = f.

PTS:	1	REF:	6

OBJ:	Know the relationship between acoustic velocity, wavelength, and frequency.

TOP:	Properties of sound waves

27.	Which acoustic variable is equal to acoustic velocity divided by the wavelength?

| a. | Intensity | c. | Frequency |
| --- | --- | --- | --- |
| b. | Pressure | d. | Period |

ANS:	C

The acoustic velocity and wavelength establish the frequency via c=fλ.

PTS:	1	REF:	6

OBJ:	Know the relationship between acoustic velocity, wavelength, and frequency.

TOP:	Properties of sound waves

28.	Which medium type is likely to have the slowest acoustic velocity?

| a. | Gas | c. | Tissue |
| --- | --- | --- | --- |
| b. | Liquid | d. | Metal |

ANS:	A

The low density and high compressibility of gas create slow sound transmission.

PTS:	1	REF:	6

OBJ:	Know the characteristics of the medium determine the acoustic velocity.

TOP:	Properties of sound waves