Frommer: Radiology for the Dental Professional, 9th Edition

Chapter 01: Ionizing Radiation and Basic Principles of X-Ray Generation

Test Bank

COMPLETION

1.	The amount of heat produced in the x-ray tube is ________ than the amount of radiation produced.

ANS: greater

REF: 15

2.	The release of electrons from the hot tungsten filament at the cathode is called the __________.

ANS: thermionic emission effect

REF: 16

3.	The determination of the size and shape of the x-ray beam is called ________.

ANS: collimation

REF: 33

4.	The quantity of x-ray photons produced is determined by the _________ setting.

ANS: milliamperage

REF: 31

5.	The quality of the x-ray photons is determined by the __________.

ANS: kilovoltage

REF: 30

6.	Because of the alternating current supplied to the x-ray tube it is considered to be _______.

ANS: self-rectified

REF: 22

7.	When designing barriers it is important to remember that x-rays travel in _______ lines.

ANS: straight

REF: 6

MULTIPLE CHOICE

1.	If the useful beam is not centered on the film in the patient’s mouth, the result will be:

| a. | foreshortening |
| --- | --- |
| b. | elongation |
| c. | collimator cutoff |
| d. | overlapping |

ANS: C	REF: 35

2.	A diagnostic film is made at 10 mA and 4 impulses. The exposure that would produce the same image at 5 mA would be:

| a. | 5 impulses |
| --- | --- |
| b. | 10 impulses |
| c. | 8 impulses |
| d. | 45 impulses |

ANS: C	REF: 31

3.	Collimator cutoff (cone cutting) is caused by:

| a. | improper horizontal angulation |
| --- | --- |
| b. | over filtration |
| c. | improper beam film alignment |
| d. | improper vertical angulation |

ANS: C	REF: 35

4.	If a radiograph is described as “underpenetrated,” then the error must be:

| a. | excessive kVp |
| --- | --- |
| b. | too little mA |
| c. | excessive mA |
| d. | too little kVp |

ANS: D	REF: 30

5.	Which of the following is used in the high-voltage circuit?

| a. | a rectifier |
| --- | --- |
| b. | a step-up transformer |
| c. | a step-down transformer |
| d. | a fuse |

ANS: B	REF: 25

6.	Ionization occurs:

| a. | when an atom gains a neutron |
| --- | --- |
| b. | when the atom has no charge |
| c. | when an atom loses a proton |
| d. | when an atom loses an electron |

ANS: D	REF: 12

7.	Identify the physicist who discovered x-rays:

| a. | Hittorf-Crookes |
| --- | --- |
| b. | Roentgen |
| c. | Edison |
| d. | Kells |

ANS: B	REF: 9

8.	Examples of particulate radiation include:

| a. | alpha particles |
| --- | --- |
| b. | electrons |
| c. | neutrons |
| d. | all of the above |

ANS: D	REF: 2

9.	The difference in the electromagnetic spectrum between x-rays and visible light lies in the:

| a. | wavelength |
| --- | --- |
| b. | frequency |
| c. | energy |
| d. | subatomic particles |

ANS: A	REF: 2

10.	All of the following are properties of x-rays EXCEPT:

| a. | x-rays are invisible |
| --- | --- |
| b. | x-rays travel in wavy lines |
| c. | x-rays can penetrate structures |
| d. | x-rays can adversely affect human tissue |

ANS: B	REF: 6

11.	Identify the particles that compose the nucleus of an atom:

| a. | electrons and photons |
| --- | --- |
| b. | electrons and protons |
| c. | protons and neutrons |
| d. | neutrons |

ANS: C	REF: 10

12.	The target or focal spot in the x-ray tube can be found at the:

| a. | positively charged cathode |
| --- | --- |
| b. | positively charged anode |
| c. | negatively charged cathode |
| d. | negatively charged anode |

ANS: B	REF: 14

13.	Identify the metal used in the focal spot of the x-ray tube:

| a. | tungsten |
| --- | --- |
| b. | molybdenum |
| c. | copper |
| d. | iron |

ANS: A	REF: 14

14.	The total x-ray energy produced by the collision of the electrons with the target inside the x-ray tube is approximately:

| a. | less than 1% |
| --- | --- |
| b. | 10%–20% |
| c. | 50% |
| d. | 99% |

ANS: A	REF: 15

15.	Pointed, plastic cones are not used in dental radiology because:

| a. | the cones are difficult for the practitioner to line up correct anatomic structures |
| --- | --- |
| b. | the cones cause the production of excess secondary and scatter radiation |
| c. | the cones require a higher kVp setting on the control panel |
| d. | the cones require an increase in the time setting |

ANS: B	REF: 28 | 29

16.	The thickness of aluminum that will reduce the intensity of the x-ray beam by 50% is termed:

| a. | half value layer |
| --- | --- |
| b. | filtration |
| c. | collimation |
| d. | penetrating power |

ANS: A	REF: 30